Chapter 5: Disassembly

Disassembling the Base Unit

These are the directions for disassembling the base unit. You will need a 6mm Nut Driver, a medium size "+" Screw driver, a chip tool and wire cutters.

These directions are to disassemble the complete unit and will be cross-referenced to Section 7 for the replacement of component parts.

Remove Battery Module

1. Press down the Battery lock button.

2. Slide the latch lock in direction of the arrow. Battery module will pop-up automatically.

3. Now, gently pull the module out of its housing.

[image: image1.jpg]



Remove HDD Module

1. Turn the power off and turn the notebook upside down with the back facing you.

2. Remove the 2 screws ( M2.5 X 0.45 + 8FD-ZK ) on the HDD module.

3. Using the ( Minus Screw Driver ) tool to pull the HDD module out of its bay.

[image: image2.jpg]



Remove CD-ROM Module

1. Remove the 3 screws ( M2.5 X 4.5P + 3FP-ZK ) on the Ram door, then pull the Raw door 
upward.

2. Remove 1 screw 【 M2.5X 0.45P + 4FP-NI(NL) 】holding the CD-ROM Module in place.

3. Push CD-ROM lock plate and remove the CD-ROM Module.

[image: image3.jpg]



Remove LCD Module

1. Remove the 2 screws (M2.5 X 0.45P + 3FP-ZK) on the back of hinge.

2. Open the notebook with the upside facing you, and then lift up the strip cover from left side to
right side.

[image: image4.jpg]



3. Pull up gently the LCD FPC Connector.

4. Remove the 4 screws fasten hinges.【Upper- (M2.5 X 0.45 + 8FP-NI(NL) X 2pcs) 
(Lower- (M2.5 X 0.45 + 18FP-ZK (NL) X 2pcs)】, and then gently lift the LCD Module.

[image: image5.jpg]



Remove Keyboard

1. Now gently lift the keyboard.

2. Pull the keyboard FPC connector safety lock in upward direction.

[image: image6.png]



Remove System window board

1. Remove the 2 screws (M2.5 X 0.45P + 3FP-ZK) on the system window board of  mother
 board.

2. Pull the FPC connector in upward direction.

[image: image7.png]



Remove Thermal Module

1. Remove the 4 screws (M2.5 X 0.45P + 3FP-ZK) and wire cable on the mother board.

[image: image8.png]2 B

-“.A.v.:-.......-.m w Ao





Remove  Mother board 

1. Remove the FPC cable on the mother board.

[image: image9.png]



Remove LCD Panel

A. For 13.3” LCD Panel:

1. Remove the 4 screws 【M2.5 X 0.45 P + 5K-ZK(NL)】 holding the LCD Bezel.

2. Then pull up the LCD Bezel.

3. Remove the 2 screws 【M2.5 X 0.45P + 5K-ZK(NL)】 of the LCD inverter.

4. Remove the 1 screw 【M2.5 X 0.45P +5K-ZK(NL) 】 holding on the LCD FPC connector.

5. Now, gently lift the LCD inverter board, and then release the right side of CCFL Connector.

6. Remove the 4 screws 【M2.5 x 0.45 +5K-2K(NL) 】on the LCD hinge bracket in the direction 
of arrows, and then remove the LCD hinge bracket gently.

[image: image10.jpg]



7. Release and pull out the FPC connector.

8. Remove the 4 screws 【 M2.5 X 0.45P + 5K-2K(NL)】on the hinge saddle kits (right & left) in 
direction of the arrow.

9. Now, remove the LCD Panel and hinge bracket gently.

[image: image11.jpg]



B. For 14.1” LCD Panel:

1. Remove the 4 screws【 M2.5 X 0.45P + 5K-2K(NL)】holding on the LCD Bezel.

2. Then pull up the LCD Bezel.

3. Remove the 2 screws 【 M2.5 X 0.45P + 5K-2K (NL)】of the LCD Inverter Board.

[image: image12.jpg]



4. Lift the LCD Inverter Board and release the right side of CCFL connector.

5. Pull the LCD FPC Holder in upward direction.

6. Separate the LCD Panel and LCD cover.

7. Remove the 4 screws【 M2.0 X 0.4P +2.3 (NL)】on the hinge saddle kits ( right & left).

8. Remove LCD FPC SET.

9. Release and pull out FPC connector in the direction of the arrow.

[image: image13.jpg]



C. For 15.0” LCD Panel: 

1. Remove the 2 screws 【M2.5 X 0.45 P + 5K-ZK(NL)】 holding the LCD Bezel.

2. Then pull up the LCD Bezel.

[image: image14.jpg]



3. Remove the 2 screws 【M2.5 X 0.45P + 5K-ZK(NL)】 of the LCD inverter.

4.  Now, gently lift the LCD inverter board, and then release the right side of CCFL connector and
 left side of LCD FPC connector

5. Remove the 4 screws 【M2.5 X 0.45 P + 5K-ZK(NL)】on the button of LCD hinge bracket.

6. Remove the 2 screws 【M2.0 X 0.4 P + 3FP-ZK(NL)】on the top of LCD hinge bracket.

7. Then pull up the LCD Panel., tear off the contact cloth on the LCD cover. 

8. Remove the LCD Panel:

   8a. pull up the LCD Panel upward.

8b. push the LCD Panel out of the hooks.

[image: image15.jpg]



9. Seperate the LCD cover and LCD Panel.

10. Remove 8 screws 【M2.0 X 0.4 P + 2.3(NL)】from both sides of LCD bracket.

11. Turn over the LCD Panel and release FPC. 

[image: image16.jpg]



Chapter 1

6
Chapter 5

7
Chapter 5


