Chapter 5
Disassembly Guide
Disassembling the Base Unit

These are the directions for disassembling the base unit. You will need a 5.5mm Nut Driver, a medium size “+” screwdriver.

These directions are to disassemble the complete unit and will be cross-referenced to Section 7 for the replacement of component parts.

Before disassembly, make sure the notebook is powered off.

Removing the Battery Pack

1. Turn the notebook upside down.

2. Unlock the battery lock. (Move the lock to the right.)

[image: image44.jpg]



3. Slide the latch lock in direction of the arrow. Battery pack will pop-up automatically.

4. Now, gently pull the battery pack out of its housing.

[image: image2.jpg]



Removing the HDD Module

Follow the steps below to remove the HDD module:

5. Turn the notebook upside down.

6. Release the two screws to release the HDD door.

[image: image3.jpg]



7. Pull up and lift the HDD door the remove it.

8. Remove the two screws that secure the HDD module.

[image: image4.jpg]



Pull out the Mylar tab to remove the HDD unit.

[image: image5.jpg]



9. Remove the four screws to take off the HDD case.

[image: image6.jpg]



10. Remove the HDD from the HDD case.

[image: image7.jpg](R0 il

_E-E.EE
A S e
BT

i i
° 303

..iu.—:n_ﬁ





Removing the DDR RAM

Follow the steps below to remove the DDR RAM:

11. Turn the notebook upside down.

12. Release the screw securing the memory module cover.

[image: image8.jpg]



13. Gently pry up the cover.

[image: image9.jpg]



14. Push the latches to release the memory module. A spring will force one end of the module up. Grasp the module and pull it out. 

[image: image10.jpg]



 Disassembling the ODD (CD-ROM/DVD-ROM/CD-RW…) Drive

To disassemble the optical drive, you need to remove the memory module cover first as described in the previous section “Removing the DDR RAM.”

Then follow the steps below:

15. Release the two screws from the bracket plate.

[image: image11.jpg]



16. Use your finger to slide out the optical drive in direction of the arrow.

[image: image12.jpg]R ™

- B 3 B ; »
- | Tbb kb |
o :
[
g kg





Gently pull out the optical drive.

[image: image13.jpg]



Removing the Keyboard

Open the display panel and follow the steps below to remove the keyboard.

17. Insert a thin tool such as a flat screwdriver into the gap between the strip cover and keyboard to lever the strip cover up. Then remove the strip cover.

[image: image14.jpg]



18. Turn over the keyboard in place, revealing the keyboard cable underneath.
Disconnect the keyboard cable from the motherboard.

19. Lift and remove the keyboard.
Removing the Thermal Module and CPU

Follow the steps below to remove the FAN, heat pipe, and CPU:

20. Turn the notebook upside down.

21. Release the two screws securing the thermal door.

[image: image15.jpg]


 

22. Gently pry up the cover.

23. Disconnect the two connectors from the motherboard as shown.

[image: image16.jpg]



24. Remove the three screws securing the FAN from the system board to remove the FAN module.

[image: image17.jpg]



25. Remove the four screws from the heat pipe. Then remove the heat pipe from the main board.
[image: image18.jpg]twyl
4 uu .__7.

)

C-a





26. Turn the cam on the CPU socket with a flat-blade screwdriver so that the notch on the cam is aligned with the open side of the CPU socket to unlock the CPU.
[image: image19.jpg]



27. Gently lift out the CPU.
Removing the LCD Module

To remove the LCD module, first remove the battery and any optional devices. Also remove the keyboard as described earlier. Then follow the steps below:

28. Release the two screws securing the LCD assembly from the rear side.

[image: image20.jpg]



29. Turn the notebook over. The keyboard should have been disassembled. If not, remove it now.

30. Disconnect the four connectors as shown.

[image: image21.jpg]



31. Release the screw securing the switch board.

[image: image22.jpg]



32. Slide out the switch board in direction of the arrows.

[image: image23.jpg]



Remove the four screws securing the LCD module to the logic upper.
[image: image24.jpg]



33. Remove the LCD module.

[image: image25.jpg]



Disassembling the Display Mask and the Inverter Board

To remove the display mask, first remove the LCD module as described in the previous section “Removing the LCD Module.” Then follow the steps below:

34. Remove the two screw pads and four screws securing the LCD bezel to the LCD module.

[image: image26.jpg]



35. Carefully insert your fingers between the display mask and the LCD bezel, and gently pry up the LCD bezel.

[image: image27.jpg]



Remove the seven screws mounting the display mask, inverter board and LVDS cable to the LCD module.

· Four screws on the display mask.

· Two screws on the inverter board.

· One screw on the LDVS cable.

[image: image28.jpg]



36. Disconnect the two connectors on either side of the board.
[image: image29.jpg]



37. Remove the inverter board.
Remove the eight screws securing the hinges to the display mask.

[image: image30.jpg]206 0k
) (
N V5w
/ ~\
206 08
) (
N Lo

7 \

0 208

Y el





38. Disconnect the LVDS cable connector from the back of the display mask.

[image: image31.jpg]


  

 Removing the Logic Upper

To remove the logic upper, first remove the battery pack, HDD, memory module, ODD, keyboard, and LCD module, as described in the preceding sections. Follow the steps below to remove the logic upper.
39. Turn the computer upside down and remove the following 18 screws from the bottom side of the notebook.
[image: image32.jpg]M}
i





40. Turn the computer over again and detach the cable as shown.
[image: image33.jpg]



41. Lift off the logic upper.

Removing the Touch Pad Board and the Finger Printer Board

To remove the touch pad board, first remove the battery pack, keyboard, and logic upper, and then follow the steps below:
42. Release the screw securing the touch pad bracket to the back of the logic upper. Disconnect the connector from the touch pad board.
[image: image34.jpg]



43. Remove the bracket.
[image: image35.jpg]ELANTECH
P/N:810511-0901 §





44. Remove the touch pad board by lifting it gently.
Removing the CRT Board, IO Board, and TV Antenna Board

The CRT board, IO board, and TV antenna board are under the logic upper. Follow the steps below to remove these three boards.

45. Remove the four screws securing the CRT board, IO board, and TV antenna board to the back of the logic upper.

[image: image1.jpg]


: One screw on the TV antenna board.

[image: image42.jpg]


: Two screws on the CRT board.

[image: image43.jpg]


: One screw on the IO board. 

[image: image36.jpg]



46. Remove the CRT board, IO board, and TV antenna board.

[image: image37.jpg]



Removing the Motherboard 

Follow the steps below to remove the motherboard:

47. Disconnect the three connectors from the back of the motherboard.

[image: image38.jpg]N
N
X
\
\
N
\
~.,,.
=





48. Lift off the motherboard.

[image: image39.jpg]



 Removing the Speaker

To disassemble the speaker, lift up the motherboard, revealing the speakers underneath. Then follow the steps below:

49. Remove the two screws securing the speaker.

[image: image40.jpg]==

2 Ll O [+

—





50. Lift and remove the speaker.

[image: image41.jpg]









